# Clutches and Brakes

- Clutches and brakes depend on friction in order to function.

Design objectives of brake and clutch design:

- Maximize friction

- Keep it uniform over a wide range of operation

- Minimize wear.

- Clutch:

The function of a clutch is to permits smooth, gradual connection and disconnection of two members with common axis of rotation.

- Brake:

- Acts the same as the clutch but one member is fixed.

- Types of clutches and brakes:

- Friction type

- Magnetic

- Eddy current

- Hydrodynamic (fluid coupling)

- In this course, we will consider only friction type brakes and clutch

- Friction Type Clutches and Brakes:

- These types of brakes must satisfy the following requirement

- The friction torque must be produced by acceptable force.

- The energy produced must be dissipated without producing high temperature.

- Reduce friction wear to give acceptable life.

## Drum Brakes

Drum brakes are of two types:

- External Shoe that contracts to bear against an outer drum surface.

- Internal Shoe that expands to contact the inner drum surface.

- Short- Shoe Drum Brake:

- The shoe contact only a small segment of drum periphery

 Short-Shoe Analysis: assume that the friction and normal force act at the center of contact.

A Free- Body diagram for clockwise drum rotation:

Taking moment about pivot A for the shoe and lever:

MA=0		⇒      Fc+fNa−bN=0					(1)

Taking moment about O of the drum:

MO=0		⇒      T=fNr						(2)

Solving  (1) for N and substituting in (2):

⇒     N=Fcb−fa

⇒     T=fFcrb−fa

- T = friction torque[ Inertial or load torque required for equilibrium]

- Self-Energizing and Self-de-Energizing Brakes:

A brake is considered self- energizing if the moment friction force (fNa) assists the applied force (F) in applying the brake (reduce the force F).

Self-energizing is produced for cw rotation of the drum.

Self-de-energizing brake: if the moment of friction force opposes the application of the brake (increase the force F required for brakeing).

Self-de-energizing is produced for ccw rotation of the drum.

Friction force opposes direction of rotation					F

T=fFcrb+fa										     fN

# ccw

# Note:

T (self-energizing) > T (self-de-energizing)

Self Locking Brake:

For cw rotation:		   T=fFcrb−fa

If :                   (b-fa) 0      no torque required to produce the brake (self-locking)

Self -Locking brake              b fa

For example: For cw rotation if  f= 0.3

Self locking       b   0.3a

  Self locking:      F =0 (no force is required to produce the brake)

Long Shoe Drum Brake:

Long –Shoe:       contact arc    45.

- Internal Shoe

- External Shoe

Internal Shoe Drum Brake:

It is normally used for automotive rear wheel .

Function:

Both shoes pivot about pin and are forced against the inner surface of the drum by a hydraulic piston at the ends. The spring is used to retract the shoes to the original position

(a)						(b)

To determine the pressure p of lining on the drum caused by applying the force F on the left shoe which is deformed (as shown by dashed line)

 = Radial deflection

δ=δ'cosθ=γAGcosα	= small angle of shoe rotation.

δ=rsinθ=γrsinθ			AGcosα=AN=rsinθ

p      sin           p= k      ,  p= pressure on the drum, k= stiffness constant of lining.

pmax           = max

⇒              psinθ=pmaxsinθmax								(1)

(sinθ)max=1.0      when    θmax=90∘        ⇒ at  (θ=90)      p=pmax⇒  max wear

 Good design: max thickness of friction material at point of max. pressure.

It is required to find the friction torque and the reactions at the hinge of the shoe.

Drum rotating cw:

Taking an infinitesimal area (dA)then integrating over the whole contact area

dA=brdθ			where:  b = face width of friction material

dN=pbrdθ										(2)

From (1):

p=pmaxsinθmaxsinθ									(3)

Substituting (3)  in (1):

⇒         dN=pmaxbrsinθ  dθsinθmax

To find the actuating force F:

1- Frictional moment about A:

Mf=fdN(r−acosθ)=fbrpmaxsinθmaxθ1θ2sinθ(r−acosθ)dθ

- Normal force Moment about A:

MN=dNasinθ=brapmaxsinθmaxθ1θ2sinθ2dθ

- Summing moment about A

MA=Fc−MN+Mf=0		⇒       F=MN−Mfc         (self-energizing)

If MN=Mf   ⇒      F=0      (Self-locking)   No actuating force is required .

In Automotive application, brake must be strongly self – energizing while staying away from self-locking.

⇒        MN≥Mf               recommended  value:  MN=(1.25−1.5)    Mf

Frictional Torque:

T=dFfr=frdN=fbr2pmaxsinθmaxθ1θ2sinθdθ=fbr2pmax(cosθ1−cosθ2)sinθmax

Reaction at hinge A:

Fh=0    ⇒       Rx=brpmaxsinθmax(A−fB)−Fx

Fv=0    ⇒       Ry=brpmaxsinθmax(B+fA)−Fy

Drum rotating ccw:

The friction force is opposite to the direction of rotation

MA=Fc−MN−Mf=0	            ⇒       F=MN+Mfc          (Self-de-energizing)

Rx=brpmaxsinθmax(A+fB)−Fx

Ry=brpmaxsinθmax(B−fA)−Fy

where:

A=θ1θ2sinθcosθdθ=12sinθθ1θ2

B=θ1θ2sin2θdθ=θ2−14sinθθ1θ2

Note: the reference is always in the center of the drum. The positive x-axis is taken through the hinge. The positive y-axis is always in the direction of the shoe.

Internal Shoe Drum Brake:

- Normally used in automotive rear wheel

- Energizing and de-energizing characteristics are determined by:

- Noting the direction of friction force acting upon the shoe surface

- Determining if this force tends to bring the shoe into or out of contact of the drum

For the left figure shown below:

Left shoe:	self – energizing

Right shoe:	self-de-energizing

In automotive brakes     self-energizing is desired      decrease pedal force

But                                     self-locking must be avoided

Improved Design:

The shoes are inverted and each one is actuated by a separate cylinder.

This arrangement will increase the self- energizing action

 Both shoes are self-energizing in forward car motion.

 This arrangement is used in automotive front wheels.

Long Shoe Drum Brake with External Shoe:

Consider the external brake shown in the figure. We have proved in previous sections that:

p=pmaxsinθmaxsinθ

- Drum rotating cw:

Summing moment about A:

MA=Fc−MN−Mf=0

⇒       F=MN+Mfc			(Self-de-energizing)

Large force is required to balance these moments (friction moment does not assist the force to produce the brake).

Where MN is the moment of normal force:

MN=dNasinθ=brapmaxsinθmaxθ1θ2sinθ2dθ=brapmax4sinθmax2θ2−θ1−sin2θ2+sin2θ1

Moment of friction force:

Mf=fdN(r−acosθ)dθ=fbrpmaxsinθmaxθ1θ2sinθ(r−acosθ)dθ

Substituting the relation: sinθcosθ=12sin2θ

Mf=fbrpmaxsinθmaxrcosθ1−cosθ2+a4cos2θ2−cos2θ1

dN

cw

r

O			fdN

Friction Torque:

T=dFfr=frdN=fbr2pmaxsinθmaxθ1θ2sinθdθ=fbr2pmax(cosθ1−cosθ2)sinθmax

Reaction forces at hinge A:

Reaction forces is obtained summing forces in horizontal and vertical direction:

Fh=0    ⇒       Rx=brpmaxsinθmax(A+fB)−Fx

Fv=0    ⇒       Ry=brpmaxsinθmax(fA−B)+Fy

where:

A=θ1θ2sinθcosθdθ=12sinθθ1θ2

B=θ1θ2sin2θdθ=θ2−14sinθθ1θ2

2. Drum Rotating ccw:

Summing moment about A:

MA=Fc−MN+Mf=0

⇒       F=MN−Mfc			(Self-energizing)

For ccw less force is desired to obtain the brake, the friction moment assists the force to produce the brake.

If    Mf≥  MN     self-energizing brake is self-locking

It is often desired to make brake self-energizing, while staying away from self-locking.

 In brake design: f≥(25−50%)    true   value,           ⇒     Mf≈  MN

Hinge Reactions:

Fh=0    ⇒       Rx=brpmaxsinθmax(A−fB)−Fx

Fv=0    ⇒       Ry=brpmaxsinθmax(−fA−B)+Fy

Symmetrical External Shoe Drum Brake:

If the pivot P is located at the intersection of the resultant normal force (N) and frictional force (fN)   the shoe will not rotate around P.

This is desirable to equalize wear.

In practice pivot P will move closer to the drum as wear occurs

 increase the tendency of the shoe to pivot around P.

Integrating over the symmetrical shoe surface, the intersection point of the normal and friction forces rf :

⇒    Mfrf=0

rf=4rsinφ2φ+sinφ

where:

rf  : is measured from the drum center. Note that  rf>r

 = total shoe angle.

With the pivot placed at this point, the moment about P is zero. The reactions at P are:

Rx=fN

Ry=N

# Friction Torque:

The friction torque, which is equal to the brakeing torque is:

T=fNrf

where: rf>r, because the resulting frictional force acting on any symmetrical shoe is located above the drum surface.

Example:

Double shoe drum brake with spring force F applied at distance c= 500 mm to both shoes. The brake is released by a solenoid. If  f = 0.3 and Pmax = 446 kPa, b=50 mm.

Determine an appropriate  value of spring force, and the resulting brake torque and power absorption for 300 rpm drum rotation in either directions.

Solution:

- Right shoe:

= tan –1(200/150)= 53.13   1 = 8.13 and 2 =98.13 .

- a=2002+1502=250 mm.

- 2  > 90 deg.,  sin max =1

- MN=brapmax4sinθmax2θ2−θ1−sin2θ2+sin2θ1

MN=50150250pmax4×12π2−sin196.26∘+sin16.26∘=−1735×103pmax,

Pmax in MPa

5- Mf=fbrpmaxsinθmaxrcosθ1−cosθ2+a4cos2θ2−cos2θ1

Mf=fpmax(50)(150)1150cos8.13∘−cos98.13∘+2504cos196.26∘−cos16.26∘=373×103fpmax

- Brake Torque

T=fbr2pmax(cosθ1−cosθ2)sinθmax=501502fpmax(cos8.13∘−cos98.13∘)1=1273×103fpmax Mo1=Fc+Mf−MN=0         ⇒       F=MN−Mfc (self-energizing)

- Fs= spring force = 1448 N

- For left shoe, the spring force will produce the brake pressure. Since, left shoe is self-de-energizing,  Pmax will be less than that for right shoe.

Mo2=Fc−Mf−MN=0         ⇒     1448500-1735×103pmax−373 ×1030.3pmax=0

pmax= 0.392 MPa

- Total braking torque

T=TRS+TLS=1273×1030.30.446+0.392=320 N.m.

- Power = T

Power=T×ω=320×3002π60=9549  watt (applies for both directions of rotation)

# Disk Clutch

Disk clutch operates dry or wet

- Dry clutch: Automotive clutch with manual transmission

- Wet clutch: multi disk clutches used in automotive automatic transmission. The oil serves as a coolant, multiple disks compensate for reduced coefficient of friction.

Estimating the axial clamping force and torque capacity is based on the two basic assumptions:

- Uniform pressure:

This assumption is valid for new, accurately manufactured clutches, with rigid outer disks.

The normal force acting on differential ring of radius r is

dF=2πrdrp

Axial Clamping Force:

⇒       F=riro2πprdr=πpro2−ri2							(1)

Friction Torque:

dT=fdFr=2πrdrpfr			f : is assumed to be constant

Total torque:

T=riro2πpfr2dr=23πpfro3−ri3           (for one driving interface)

Multiple Clutch interfaces:

For N friction interfaces transmitting torque in parallel   Total torque capacity:

T=23πpfro3−ri3N								(2)

From eq. (1):

p=Fπro2−ri2									(4)

Substituting (4) in (2):

T=2fFro3−ri33ro2−ri2N									(5)

- Uniform Wear:

Wear rate      Friction Work rate

Friction Work = Friction Force  Rubbing velocity

For constant f     Wear    pressure  Sliding velocity       Wear   p ve

For disk clutch:

       ve    r                                    Wear       p  r

    Uniform Wear results from uniform rate of friction work    pve=C

- pr=C

   pmax   at  rmin

For disk clutch with:  ro = outside radius, ri = inside radius

     pmax   at    r = ri

pr=C=pmaxri

⇒       F=riro2πpmaxridr=2πpmaxriro−ri

T=riro2πpmaxrifrdrN=πpmaxrifro2−ri2N						(6)

⇒      T=fFro+ri2N

Assumption of uniform wear gives lower clutch capacity than uniform pressure.

Clutch Design: Clutches are designed for uniform wear.

From eq. (6): Max. torque for a given outside radius is:

ri=13ro=0.58ro

Recommended Range: ri=    0.45ro  −   0.8ro

Disk Brake:

A brake is similar to clutch except that one of the shafts is replaced by a fixed member.

This type of brakes is unsatisfactory because of inadequate cooling.

For this reason, caliper disk brakes are commonly used.

Drum brake can be designed for self-energizing this has an advantage and disadvantage:

Advantage: reduced braking effort

Disadvantage: slight change of friction coefficient (as a result of temperature rise or moisture) will cause a large change in pedal force required for braking.

Disk brakes has no self-energization, hence is not sensitive to changes in friction coefficient.

Caliper disk brake have replaced front drum brakes due to:

- Greater cooling capacity

- High resistance to fade

- Drum brake tends to distort more with heat and large forces than disk brakes.

Cone Clutch and Brakes:

Cone clutch and brake can be considered as the general case of which disk brake is a special case, the cone angle = 90.

Cone clutch normally has one friction face   N=1

The effect of the cone angle is to reduce the clamping force.

For the cone clutch shown on the figure:

dA=2πrdr/sinα

Normal force on dA:

dN=pdA=2πrdrp/sinα

Axial Clamping force:

dF=dNsinα=2πrdrp

Friction Torque:

dT=dNfr=2πpfr2dr/sinα

- Uniform pressure:

F=riro2πprdr=πpro2−ri2

T=23πpfro3−ri3sinα

T=2fFro3−ri33ro2−ri21sinα

- uniform Wear:

F=2πpmaxriro−ri

T=πpmaxrifro2−ri2sinα

T=fFro+ri2/sinα

Note:
The smaller     the lower the clamping force  the higher the friction torque.

If   < 8    the clutch tend to grab and become difficult to disengage.

   8<< 15

Optimum value: =12.

Band Brakes:

The band is made of steel, lined with a woven friction material

For cw drum rotation:

Friction force increases P1 and decrease P2 ,         P1>P2

Free-body diagram of Drum:

T=(P1−P2)r

Free-body diagram of Lever:

F=aP2c

Forces acing on an element of the band:

For small angle d

Fh=0

  dP=fdN									(1)

Fh=0

  dN=2pdθ2=Pdθ							(2)

Substituting (2) in (1):

     dP=fPdθ

     dPP=fdθ

Band force varies [P1-P2] between band contact angle  =[0 - ]

P2P1dPP=f0φdθ                lnP1P2=fφ		P1P2=efφ

But:

dN=pdA=pbrdθ 		, where:     p = pressure between drum and band.

dN=Pdθ

  P=pbr

  P    p

pmax  occurs at Pmax = P1

at     =        P= Pmax = P1

  at     =              P1=pmaxbr

For cw rotation: Self-energizing

F=aP2c           ,   where     P2 < P1      [friction force increases P1 and decreases P2]

For ccw rotation: Self-de-energizing

F=aP2c     	, where          P1 < P2     [friction force increases P2 and decreases P1]

Differential Band Brake:

To increase the self-energizing action the differential

F=P2a−P1sc  ,  where P1 < P2  due to friction force

For self energizing: s < a   (to insure that the lever will tighten the band at distance a more than at distance s).

Energy Consideration in Brakes and Clutches:

The hoist shown in the figure is required to lower a mass (m)

Assume that the brake is applied at time = t1

At  t1

 = 1		, V= V1		, h = h1

At  t2    the values are reduced

 = 2		, V= V2		, h = h2

during time (t2 – t1)

# Work done by brake = wkB

# Rolling friction, Bearing friction, air resistance = wkR

Motor work = wkM

wkM  = Work done by the machine in driving energy consuming device.

If the motor drives the machine during braking period  wkM = -ve

# Total work done = Change in Energy

wkB+wkR+wkM=m2(V21−V22)+I2ω12−ω22+Wh1−h2

the summation is made for different masses at their corresponding speeds and positions.

wkB = mechanical energy transferred into heat at the brake .

wkB=ψ1ψ2Tdψ  =T(ψ2−ψ1)

 = angular displacement of the drum

For constant torque:

If 	T= const.    linear deceleration = a= const.

- Angular deceleration =  = const.

Velocity displacement relation:

V2=V1−a(t2−t1)

S2=S1+V1(t2−t1)−a2t2−t12=S1+V1+V22t2−t1

and angular velocity and displacement:

ω2=ω1−α(t2−t1)

ψ2=ψ1+ω1t2−t1−α2t2−t12=ψ1+ω1+ω22t2−t1

Clutch Work Energy Relationship:

In the clutching of two masses initially rotated in the same direction with different speeds, the higher speed mass supplies energy to lower speed mass.

In general: the work done by friction equal the net change in energy of the system.

Consider two masses with I and I, rotating in the same direction with initial velocities > 

# Assuming that clutch torque , T= const. during engagement

At time,  t1=0

> 

Left Shaft:

T=−Idωdt

Right Shaft

T=I'dω'dt

Integrating both equations:

ω=ω1−TIt

ω'=ω1'+TI't										(1)

A t=t2 , relative motion ends (both shaft rotate at the same speed)

 2 = 2

ω1−TIt2=ω1'+TI't2

t2=ω1−ω1'TII'I+I'

  the time required for engagement is proportional to velocity difference

t2 = time required for clutch engagement

Subst. t2 in eq. (1)

    Common velocity:

ω2=ω2'=ω1I+ω1'I'I+I'

Displacement after time t2:

ψ=0tωdt=0tω1−TItdt=ω1t−Tt22I

ψ'=0tω'dt=0tω1'+TItdt=ω1't+Tt22I'

The work done by the clutch after time t:

# Work = Clutch Torque Relative displacement

wK=T(ψ−ψ')=Tω1−ω1't−T2I+I'2II't2

Clutch work at completion of engagement, t = t2

wKC=II'2I+I'ω1−ω1'2

wkc = energy dissipated by the clutch or brake.

 energy dissipated is proportional to velocity difference squared.

S. I. units:

wkc = E = Joule

U. S. units:

wkc = E = lb.in., wkc = E = (lb.in./9336)= Btu.

Energy Absorption and Cooling:

Heat transfer equation of clutch or brake:

H=CAΔT=CA(Ts−Ta)

H = time rate of heat dissipation (Watt or hp)

C = overall heat transfer coefficient per unit area (W/m2 C, hp/in.2 F)

A = exposed heat –dissipating area (m2 , in2)

Ts = average temperature of heat-dissipating surfaces (C, F)

Ta = Air temperature

Or

E=mCΔT=mC(Ts−Ta)

E = heat dissipation ( Joule or lb.in)

C = specific heat (J/m2 C, lb.in/in.2 F)

m = mass of brake or clutch (lbm, Kg), (Btu = 9336 lb.in)

For steel and Cast Iron: C = 0.12 Btu/(lbm. F) = 500 J/(Kg. C)

The rate of heat generation per unit area of friction interface is equal to the product of Clamping pressure, coefficient of friction and rubbing velocity

H= f pV

An emperical values for pV is obtained by brake manufacturer and is given below:

| Operating Conditions | PV |  |
| --- | --- | --- |
|  | (psi) (ft/min) | (KPa)(m/s) |
| Continuous poor heat dissp. | 30,000 | 1050 |
| Occasional, poor heat dissp. | 60,000 | 2100 |
| Continuous good heat dissp. | 85,000 | 3000 |

The ability of brake and clutch to absorb energy without reaching destructive temperatures can be increased by:

- Increasing exposed surface area

- Increasing air flow by minimizing air flow restrictions, and maximizing air pumping action of rotating parts.

- Increasing the mass and specific heat of contact parts.

Example:

An inertial load is to bring to rest in 10 sec. by a braking torque 2800 lb.in. The initial speed is 1600 rpm, and the heat dissipating surface weight 40 lb. Find the temperature rise.

Solution:

ω1=2πn60=2π160060=168 rad/s

2 = 0

but: t2=ω1−ω1'TII'I+I' = 10 sec

II'I+I'=Tt2ω1−ω1'=2800(10)168=166.7 lb.s2. in.

wKC=II'2I+I'ω1−ω1'2=166.716822=2.35(106)  lb.in.

H=mCΔT

H= 2.35(106)/9336 = 252 Btu

T= 252/[0.12(40)]=52.5 F

Friction Material:

One of the mating material is normally metal (cast iron or steel) it must have:

- Good friction characteristics ( which is relatively stable over temp. range)

- Good thermal conductivity

- Good resistance to wear and thermal fatigue

- mooth surface finish to minimize wear of the mating friction material.

The friction material must have the following requirements:

- High dynamic coefficient of friction that is relatively stable over a usable temp range and not sensitive to moisture, dirt and oil

- Static coefficient of friction exceeds the dynamic value by as little as possible to avoid slip-stick chatter and noise.

- High resistance to abrasive and adhesive wear

- Good thermal conductivity

- Good temperature stability or resistance to fade (reduction in friction coefficient at elevated temp.)

Types of friction material:

- Molded: most common and least costly. Used for heavy duty application and drum brakes. Molded asbestos contains asbestos fiber and friction modifiers

- Woven: more flexible (suitable for band brake)

Asbestos lining is used for heavy machinery, perform better when contaminants as mud, grease, and dirt presents.

- Sintered: made of mixture of copper and iron particles with friction modifiers. Used in clutch and brakes for heavy duty applications.

Tables (16-1,2) list properties of typical friction materials

Example:

The figure shows a band brake used with a punch press. The brake is to be engaged when the crank is 120 past bottom dead center and bring the crank to rest at top dead center. The crank assembly has I= 12 N.m s2 and rotating at 40 rpm when the brake is engaged. Max. allowed pressure p= 0.18 MPa, f = 0.35.

- Determine the required band width.

- Determine the required force, F

- Would any combination of direction of rotation or coefficient of friction make brake self-locking.

Solution:

(a) Work done by the brake = Change in K.E. of the system

ΔK.E.=12Iω12−ω22=12(12)(40×2π60)2=105.275 J

Brake done by brake = Torque angular displacement

wkB=T(ψ2−ψ1)

(ψ2−ψ1)=180−120=60∘=1.047  rad				Top dead center

ΔK.E.=T(ψ2−ψ1)

T=ΔK.E(ψ2−ψ1)=105.2751.047=100.56  N.m                                             120

Free body of the drum:						bottom dead center

T=(P1−P2)r

    (P1−P2)=Tr=100.560.12=837.92  N						(1)

But:

P1P2=efφ

= 258,   f = 0.35		P1P2=e0.35×258×π180=4.835 				(2)

From (1) and (2):

P1 = 218.41 N,  P2 = 1056.16 N

P1=pmaxbr		b=P1pmaxr=1056.160.18×1060.12=49 mm

(b) Free body of the lever

Mo=F(370+55)−P2(72)−P1(55)=0	   F = 173.7 N