Birzeit University

Mechanical & Mechatronics Engineering Department

Thermodynamics ENME 333

Midterm Exam Part 1

Dr. Afif Akel						Second semester 2020-2021

Instructions: Closed book exam open tables, equations are provided. No interpolation is needed may use approximate property values.

Form A

Problem 1 (30%)

Determine the phase, the quality if applicable, and the missing properties. Give the details of your solution.

- Water at 100kPa, 40oC 		u=  ?,	h=?	v=?

- Ammonia 40oC, v= 0.05 m3/kg	u=?,	h=?	P=?

- Air at 110kPa, 27oC			u=?,	h=?	v=?

Problem 2 (30%)

Air enters an insulated jet engine nozzle at 10m/s, 200kPa, 400K and leaves at 200m/s, 100kPa.

- List assumptions to solve problem [4]

- Find exit air temperature [12]

- Show process on P-v diagrams [3]

- If inlet diameter is 80 cm what is the volume and air mass flow rates?[7]

- Find exit diameter? [4]

Form B

Problem 1 (30%)

Determine the phase, the quality if applicable, and the missing properties. Give the details of your solution.

- Water at 100kPa, 150oC 		u=  ?,	h=?	v=?

- Ammonia 70oC, v= 0.001m3/kg	u=?,	h=?

- Air at 150kPa, 400K			u=?,	h=?	v=?

Problem 2 (30%)

Air enters an adiabatic diffuser at 100m/s, 50kPa and leaves at 10m/s, 200kPa, 310K.

- List assumptions to solve problem [4]

- Find inlet air temperature [12]

- Show process on P-v diagrams [3]

- If inlet diameter is 10 cm find volume and mass flow rates? [7]

- Find exit diameter [4]

Form C

Problem 1 (30%)

Determine the phase, the quality if applicable, and the missing properties. Give the details of your solution.

- Water at 2.0MPa, 200oC 	u=  ?,	h=?	v=?

- R134a; 10oC, u=300kJ/kg	v=?,	h=?	P=?

- Air at 0. 2MPa, 47oC		u=?,	h=?	v=?

Problem 2 (30%)

Air enters an insulated jet engine nozzle at 20m/s, 300kPa, 600K and leaves at 400m/s, 150kPa.

- List assumptions to solve problem [4]

- Find exit air temperature [12]

- Show process on P-v diagram [3]

- If inlet diameter is 150cm calculate air volume and mass flow rates? [7]

- Find exit diameter? [4]