Development of microstructure in Isomorphous alloy in Equilibrium alloy

Binary Eutectic system

The eutectic reaction is transition from a liquid phase to two other different solid phases

L                             α+β

Eutectic is  a Greek word which means easy to melt which means that the alloy resulted out of this reaction the melting point of it is lower than either of the alloying components

Solvus line separates one solid solution from a mixture of

solid solutions. Solvus line shows limit of solubility

Eutectic or invariant point - Liquid and two solid phases

co-exist in equilibrium at the eutectic composition CE and

the eutectic temperature TE.

Eutectic isotherm - the horizontal solidus line at TE.

Led-tin phase diagram

Development of the microstructure in eutectic alloy

During slow cooling	led-tin

L 	α+L         α

Development of microstructure –continue

Development of microstructure of eutectic alloy

As we know the composition of α and β are very different there is a redistribution of  Pb and Sn atoms the formation of α and β will be in the form of layers microstructure and this is called eutectic structure

The dark layer is led Rich α phase and the light layer is tin rich β phase

Development of microstructure –continue

Primary α is formed in the region of α+L phase

The eutectic structure is in the phase of α+β and is called eutectic α and eutectic β and it is formed when we are below the eutectic temperature

And this structure is called microconstituent and is a unique structure consists of two microconstituent primary α and the eutectic structure

How to calculate the relative amount of the microconstituents

How to calculate the total amount of α phase both in eutectic and primary

How to calculate the total amount in α phase both primary and eutectic