Birzeit University

Physics department

Physics 211

Student’s name: Ahmad Hammouri 	Student’s number: 1071811

Partners name: Anas Abu halaweh 	partner number:

Partner’s name: Mariana Mosallam 	partners number: 1100343

Experiment number: (4)

Experiment name:  torsional torques and torsional modulus

Date: 3/2/2012

Instructor: Dr. Wael .Q

Abstract:

The torsional  pendulum is used to study the elastic properties of aluminum and steel rods  the torsional  constant is determined  and its dependence on  the rods geometry (n,m ) was  studied  also the shear modulus (G) was determined .

The results were:

I=0.0110± 0.002 Kg m2

m=0.90 ± 0.13-1

n= 4.3 ±0.54

G(avg) = (23.7± 1.0)x10^9 (N/m2)

Apparatus:

A  massive dumbbell shaped object is fixed  to a thin metallic rod (aluminum or steel  ). The system  is twisted  and set in vibration . The period  for small vibrations is measured  and is related  to the torsional constant

Theory:

The period  for small oscillations  is given by :

T= 2πIK

Where T is the  period ,I is  the moment of inertia of the system ,and k is the torsional constant .Note  that the mass of the rod is small compared with that of the dumbbell  , and I therefore is constant  for the whole experiment and is determined once .(Irod is negligible )

For elastic twisting  of the rod , the torque τ is related to the twist angle Ѳ by:

Τ=-ҡѲ

Where Ҡ  itself is related to  the dimensions of the rod by the following  relation:

Ҡ= GπLmdn32

In this last formula,G is  the shear  modulus , d is the rods diameter ,and L is the length .

Procedure:

Step one :Determine  the moment of inertia  of the apparatus as follows :

- Using one rode (picked at random ), twist the system through six different  angles Ѳ calculating τ each time .plot τ vs Ѳ and use the graph to  determine Ҡ .(to calculate the torque the applied force should be read  from spring used to pull the dumbbell object )

- Set the dumbbell  object  into oscillation  by pulling  it with the spring and releasing it .Measure the period of oscillations for the system and determine I .

- This value of I is constant throughout the experiment.

Step two: ҡ vs. d (the rods diameter).

- Take the aluminum bars having the same length.

- Perform small oscillation experiments on each determining the period for each rod.

Step three: repeat step two with fixed d and L varied.

Data analysis:

Part 1: determination of moment of inertia of the system:

| Ѳ (degres) | Ѳ(radians) | F (N) | τ(N) |
| --- | --- | --- | --- |
| 10 | 0.174533 | 0.5 | 0.025 |
| 20 | 0.349066 | 0.78 | 0.039 |
| 30 | 0.523599 | 1.2 | 0.06 |
| 40 | 0.698132 | 1.8 | 0.09 |
| 50 | 0.872665 | 2.45 | 0.1225 |

Diagram used to find the torsional constant ҡ:

Slope = ҡ, so ҡ =0.1409

Y intercept = 0.0065 experimentally while it ought to be zero theoretically.

∆Ѳ= 0.017453 rad

∆τ= 0.05 N

∆ ҡ= 0.001264

So ҡ =0.1409 ± 0.0013 (Nm/rad)

For determination of I

T= 2πIK =1.8s

- I=(T/2π)2* K

I=0.01157541  ± 0.0018851344

I= 0.011 ± 0.002 Kg m2 (same throughout the experiment)

Part 2: ҡ vs. d (the rods diameter). Determination of m and G1

The slope of the log K vs. log L diagram is m

m= -0.9007 ± 0.1254  m≈-1

Ҡ= GπLmdn32

| L(m) | T(s) | K(Nm/rad) | logL(m) | log K(Nm/rad) | G (N/m2) |
| --- | --- | --- | --- | --- | --- |
| 0.291 | 1.8245 | 0.130324 | -0.53611 | -0.88498 | 24155585785 |
| 0.398 | 2.0201 | 0.106308 | -0.40012 | -0.97343 | 26949434408 |
| 0.496 | 2.331 | 0.079841 | -0.30452 | -1.09777 | 25223738171 |

Part 3: calculation of the dependence  of the torsional modulus on the diameter of the rod (n):

|  | d(m) | T (s) | k | log d | log K | G N/m2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.00405 | 0.5905 | 1.248833 | -2.39254 | 0.096504 | 23746983305 |
| 2 | 0.0031 | 1.0273 | 0.412619 | -2.50864 | -0.38445 | 22857408615 |
| 3 | 0.00215 | 2.316 | 0.081183 | -2.66756 | -1.09053 | 19437365173 |

The slope of the diagram =4.3238 also = n

So n= 4.3238 ±0.514

n≈4

Results :

I=0.0110± 0.002 Kg m2

m=0.90 ± 0.13-1

n= 4.3 ±0.54

G(avg) = (23.7± 1.0)x10^9 (N/m2)

Discussion of results:

The sources of error :  the aluminum bars we studied were not perfectly  straight which could affect  because they were bend which would lower the  length affecting the torque

The timer also can give such systematic error, and the human eye too.

Conclusion :

the dependence  of the torsional modulus on the diameter of the rod (n) is in the order of 4 and the dependence  of the torsional modulus on the length of the rod (m) is in the order of -1

so the mathematic equation Ҡ= GπLmdn32 becomes Ҡ= GπL-1d432