Birzeit University

Mechanical & Mechatronics Engineering Department

Thermodynamics ENME 333

Midterm Exam Part 2

Dr. Afif Akel						Second semester 2020-2021

Instructions: Closed book exam open tables, equations are provided. No interpolation is needed may use approximate property values.

Form A

Problem 3 (40%)

A piston /cylinder containing 50 kg of water with a volume of 1.2 m3 at 600kPa. System is heated isobarically to 300 oC

- Determine the initial specific volume, phase, and temperature of water? [6]

- Calculate the final volume? [7]

- Describe the heating process then show process on T-v and P-v diagrams [6]

- Calculate the work [7]

- Calculate the heat transfer [14]

Form B

Problem 3 (40%)

A piston/cylinder device containing 150 kg of R134a at 20 oC with a volume of 0.8 m3 .System is heated isothermally to a final volume of 1.8 m3.

- Determine the initial specific volume, phase and pressure [6]

- Determine the final phase, pressure [6]

- Describe heating process and show on T-v and P-v diagrams? [7]

- Calculate the work [7]

- Calculate the heat transfer [14]

Form C

Problem 3 (40%)

A cylinder fitted with a piston contains 1.5 kg of water at 100 kPa and 0.75 m3 volume. Water is heated isothermally till volume becomes 1.5 m3

- Determine the initial specific volume, phase and temperature [6]

- Determine the final phase, and pressure [6]

- Describe heating process and show on T-v and P-v diagrams? [7]

- Calculate the work [7]

- Calculate the heat transfer [14]