Faculty of Engineering & Technology – Electrical & Computer Engineering Department

Second Semester 2021 – 2022

Communication Laboratory ENEE4113

# Experiment 2 - Prelab

# Name: Ahmaide Al-Awawdah.

# ID: 1190823

# Section: 9

# Instructor: Dr. Ashraf Al-Rimawi

TA: Eng. Ruba Eid

# Date:  1st April 2022

m(t) = cos(2π1500t),  c(t) = 4 cos(2π100000t)

- SSB-SC modulated signal = Ac m(t) cos (ωc t) – Ac mh(t) sin (ωc t)

mh(t) = cos(2π1500t + π2),

- The figure below shows the plotted signal using MATLAB software.

### Figure 1: SSB-SC modulated signal

The code

Am = 1;

fm = 1500;

Ac = 4;

Fc = 100000;

t=-0.0001:0.00000001:0.0001;

m = Am.*cos(2*pi*fm*t);

d90 = pi.*0.5;

M_shift = Am.*cos(2*pi*fm*t + d90);

s = Ac.*m.*cos(2*pi*Fc*t) - Ac.*M_shift.*sin(2*pi*Fc*t);

plot(t, s,'-');

axis([-0.0001, 0.0001, -5, 5]);

xlabel("Time"), ylabel("Signal");

title("SSB-SC modulated Signal");

The block diagram

### Figure 2: SSB-SC modulated signal block diagram